Release note

Contents

Release 1.1.15.6	2

Fixed bugs	2

Improvements	2

New features	2

Release 1.1.14.10	3

Major information	3

Fixed bugs	3

Release 1.1.13.4 (ONLY XVP222S and XVG222S)	4

Major information	4

Fixed bugs	4

Release 1.1.0.11	4

Major information	4

Fixed bugs	4

Know issues	5

Release 1.1.0.9	5

Major information	5

Release 1.1.0.8	5

Major information	5

# Release 1.1.15.6

## Fixed bugs

- ALL – 96 [OCPP] : GetDiagnostics files : Convert the enum values in readable text (example: IEC 1 states : Available, IEC 2 states : Available instead of IEC 1 states : 0, IEC 2 states : 0)

- ALL – 157 [OCPP] : Updates with OCPP. An incorrect link for updating the wallbox via OCPP causes the wallbox to ignore all subsequent update requests.

- ALL – 162 [Webapp] : "Remember me" feature does not work in the login page

- ALL – 170 [OCPP] : If content of HagerSupportScreenQrCodeContent is empty, nothing is displayed on screen if a call number is added in HagerSupportScreenPhoneNumber key

- ALL – 175 [Network] : Static IP address, APN and PIN code are not reset to default values after a factory reset

- ALL – 188 [SWID] : Help number displayed on 2 lines when too long

- ALL – 193 [Input] : Disable input does not work

- ALL – 194 [Input] : Reboot of the charger is required to apply input configuration

- ALL – 206 [OCPP] : Status Notification Finishing not sent if SuspendEV/SuspendEVSE is triggered

- ALL – 228 [SWID] : SWID is blocked on "Starting charging" screen when access mode is in free access and full EV is plugged on the charger

- ALL – 272 [OCPP] : OCPP Faulted status not reported if we are not in a transaction

- ALL – 278 [4G] : Not possible to enter a pin code with 6 digits

- ALL – 281 [OCPP/LLM] : Unlock connector failed in suspended EVSE when stopping the charge by pressing the button and then passing the badge

- ALL – 306 [Update] : Firmware update via OCPP or through the Webapp stop the current charge

- ALL – 311 [OCPP] : When a charge is in progress on Connector 1 and 2 : Offline messages are sent multiple times

- ALL – 312 [OCPP] : When a charge is in progress on Connector 1 and 2 : Offline messages are sent in wrong order

- ALL – 313 [OCPP] : During a firmware update, the status of connector must not be “Faulted”

## Improvements

- ALL – 129 [UI] : When a CPO address is configured, force Access method to "Controlled by badge" . After CPO deletion, or factory reset, reactivate "Free access“.

- ALL – 298 [OCPP] : Change ErrorCode when Overconsumption is triggered 4 times in a row 3 faulted status are OverCurrentFailure and the last one is OtherError

- ALL – 321 [OCPP] : Implement OCPP Key ( "CustomDisplayCostAndPrice", boolean, default: "false" , RW). CPO Chargepoint is checking if this key is here to send the tarif and cost features

- ALL – [4G] : Compatibility with all the 4G SIM cards with VPN

## New features

- ALL – [OCPP] : Tarif display on the screen. Settings through ocpp (Final cost, default price from the OCPP & California Pricing Requirements v3.1, 13-09-2024).

# Release 1.1.14.10

## Major information

- Compatible with LLM in auto configuration for XVx222SQF and XVx222SQD (min LLM version 2025_332).

## Fixed bugs

- FR 7kW – 183 [OCPP] : Wrong Meter type in BootNotification

- FR 7kW – 184 [OCPP] : Wrong L2 and L3 information are sent in Measurand of MeterValues when charging on connector 1 or 2

- FR 7kW – 185 [OCPP] : Wrong power offered value in Power.Offered measurand

- DE – 258 [Eichrecht] : Timestamp of the StartTransaction and Stoptransaction and Metervalue transactionEnd are 50s in the future.

- DE – 263 [OCPP] : Charge is stopped when the EV is in Suspended EV for one minute

- DE – 264 [OCPP] : Wrong status sent when SuspendedEVSE

- CE/DE – 271 [OCPP] : CE/DE charger with 2 connectors. Charger reboot when we send ChangeAvailability Operative/Inoperative on connector 3.

- ALL – 273 [OCPP] : Offline messages: if a charge is in progress offline, the offline metervalues are stored, when the charger is online again the present metervalues are also sent in between.

- ALL – 274 [OCPP] : Offline messages: some metervalues are sent twice, and stop transaction is not queued.

- ALL – 276 [network] : When switching in 4G, hostname via hotspot no more usable. Forced to use hager.local or ip

- ALL – 279 [network] : No Mdns after a reboot with power cut

- FR 7kW – 282 [OCPP] : Metervalues not sent on connector 2

- FR – 284 [OCPP] : follow the specification "Multiple Connectors per EVSE“ in OCPP 1.x implementations v1.2 Final, 2024-03-25

- FR 7kW – 285 [OCPP] : Send charging Profile with Number of phase to 3 is accepted by the charger

- FR 7kW – 286 [SWID] : Wrong Power offered/available displayed on the display for M3 connectors

- FR – 287 [OCPP] : Stoptransaction issue : the Stoptransaction of connector id 3 is not sent before the reboot

- FR – 289 [OCPP] : Connector 3 Available status not sent

- FR – 294 [OCPP] : meterstop is 0 in case of Powerloss

- ALL – 297 [OCPP] : Connector ID3 status notification sent during the update on a CE version

- ALL – 299 [network] : interface br0 and mdns reflector not activated by default

- ALL – 301 [Webapp access] : Not possible to access webapp when connected to hotspot wifi using https://hager-evcs-xxxxxx, only https://192.168.3.5 is working

- FR – 302 [OCPP] : Connector ID 3 status is Unavailable after update from V1.1.12.4 to V1.1.14.4

- ALL – 309 [OCPP] : QrCode keys removed after update

Improvements

- DE – 262 [Eichrecht] : Implement 2 new OCPP keys : MeterPublicKey1, MeterPublicKey2, Content is the public key encoded base 64 available for DE version only

- DE – 268/269 [Eichrecht] : Change Eichrecht OCMF data

- ALL – 288 [WebApp] : Router software version is display in the webapp

# Release 1.1.13.4 (ONLY XVP222S and XVG222S)

## Major information

- Compatible with LLM in auto configuration.

## Fixed bugs

- 170 : SetChargingProfile rejected with LLM

- 174 : Connection to the LLM

- 177 : Sending command with too many characters makes the wallbox to ignore message and sometimes to crash OCPP until reboot

- 179 : Metervalue Limit is not enough (offline mode)

- 206 : Status Notification Finishing not sent if SuspendEV is triggered

- 212 : When hotspot is disabled by the wallbox, the WebApp is not turned off

- 216 : "RemoteStart with id tag with more than 19 characters are truncated by the EVCS in the StartTransaction

- 223 : Offline metervalues are not sent

- 225 : Missing BootNotification at startup (DE product)

- 229 : "When the charger is connected to OCPP server 1 and we change the URL in the webapp to OCPP server 2 and save. The charger is not connecting to the new server. A reboot is necessary"

- 235 : "When the car is in Suspended EV, we stop the charge using the RFID=> No unlock of the socket, we need to unplug EV side first"

- 239 : The Bootnotification is sent two times

- 241 : no connection of the charger to the LLM if internet connection is lost

- 242 : No unlock of the Connector when charger is configured Free Access and CPO configured

- 246 : "Unlock connector failed in suspended EV 2nd Unlock is applied correctly"

- 274 : "Offline messages:some metervalues are sent twice, and stop transaction is not queued."

# Release 1.1.0.11

## Major information

- Compatible with LLM in manual configuration only

## Fixed bugs

- 14 : The power of the 1P car is reduced when connecting and charging with a 3P car afterwards.

- 132 : 4G connection configuration

- 149 : Input active in loadshedding at 0A stopped the charge

- 150 : Overconsumption from the EV in charge cause a stop of the charge

- 165 : If static IP is set, it’s not possible to connect the OCPP server.

- 181 : StopTransaction not received on Connector 1 after a Powerloss / or Hard reset

- 197 : After reset command the charger is disconnecting and sending Bootnotification two times then disconnecting again.

- 173 : Adding a "/" character at the end of a CPO's address may cause connexion issue

- Impossible to start charging if the user first connects the cable on car side (in free access mode)

## Know issues

- 206 : Status Notification Finishing not sent if SuspendEV is triggered

- 216 : RemoteStart with id tag with more than 19 characters are truncated by the EVCS in the StartTransaction

- 223 : Offline metervalues are not sent

# Release 1.1.0.9

## Major information

- Not compatible with LLM.

- Not 100% of the functionalities are implemented.

# Release 1.1.0.8

## Major information

- Witty Park 2 launch (XVG222S and XVG222SQF). Not compatible with LLM.

- Not 100% of the functionalities are implemented.